# BELL FIRE EQUIPMENT COMPANY

12-14 Yookson Road Picton Industrial Area Po Box 9334 PICTON

Western Australia 6229

PTY LTD

www.bellfire.com.au

Telephone: (08) 9725 6100

Fax: (08) 9725 6105

Email: admin@bellfire.com.au

ABN: 2800 9271 961

Pamphlet No 15

BELL FIREFIGHTER

EQUIPMENT RANGE

September 2022

# BLACKOUT DIFFUSER NOZZLES

AUSTRALIAN MADE RANGE OF DIFFUSER STYLE NOZZLES DEVELOPED AND RUGGEDLY BUILT PREDOMINATELY FOR BUSHFIRE CONTROL.

FITTED WITH A UNIQUE FLOATING DIFFUSER WHICH PREVENTS DEBRIS INDUCED WATER STREAM DISTORTION. BLACKOUTS ARE MOULDED RUBBER OR P.E. BUMP PROTECTED AND HAVE MINIMAL ROTATIONAL TURNS BETWEEN JET / SPRAY FLOW CHARACTERISTICS FOR EASE OF GLOVED HAND USE.

AVAILABLE IN SIZES TO SUIT HOSE REEL AND LAYFLAT USE AND UTILISED IN A VARIETY OF ENGINEERED PRODUCTS SUCH AS PISTOL GRIP NOZZLES, HI-VOLUME MONITOR DISCHARGE, FIXED FIRE PROTECTION DELUGE AND DIRECT COUPLED PLANT WASHDOWN.

FD/20

FD/40

FD/25

PISTOL GRIP NOZZLES

See page 3

## BLACKOUT NOZZLE PERFORMANCE TESTS

EXPRESSED IN LITRES / MIN AT LISTED PUMP PRESSURES

|  | FD/20 @ L/Min |  | FD/25 @ L/Min |  | FD/40 @ L/Min |  |
| --- | --- | --- | --- | --- | --- | --- |
| PRESSURE | JET | FOG | JET | FOG | JET | FOG |
| 5 BAR 75 PSI | 75 | 170 | 85 | 180 | 215 | 450 |
| 7 BAR 105 PSI | 90 | 215 | 140 | 270 | 340 | 710 |
| 10 BAR 150 PSI | 105 | 245 | 160 | 325 | 410 | 950 |

## NOZZLE SPECIFICATIONS

| NOZZLE (Stock Number) | FD/20 (36840) | FD/25 (36855) | FD/40 (36860)* |
| --- | --- | --- | --- |
| OPERATION | 1 x BUMP RING ROTATION | 2/3 BUMP RING ROTATION | 2/3 BUMP RING ROTATION |
| SPRAY PATTERN | FOG SPRAY/JET/OFF | FOG SPRAY/JET/OFF | FOG SPRAY/JET/OFF |
| LENGTH | 118mm | 125mm | 158mm |
| O/A WIDTH | 50mm | 75mm | 100mm |
| WEIGHT | 0.260kg | 0.580kg | Alum – 0.740kg Brass – 1.850kg |
| CONNECTING THREAD | - 20mm BSPM - 20mm Brass adaptor to 20mm BSPF - 20mm Brass tail | - 25mm BSPF - 25mm Brass tail - 25mm / 20mm Brass tail | - 40mm BSPF |

MATERIALS

| BODY - | UNC MACHINED BRASS BAR STOCK. |
| --- | --- |
| SHELL - | INJECTION MOULDED BLACK UHMWPE. |
| BUMP RING - | BLACK MOULDED RUBBER. (FD/20 – P.E.) |
| *FD/40 Body available in Brass or Anodised Aluminium |  |

# BLACKOUT PISTOL GRIP NOZZLES

BLACKOUT FD/20 PISTOL GRIP NOZZLE

WITH 25MM SWIVEL WITH STORZ 25MM COUPLING

BLACKOUT FD/25 PISTOL GRIP NOZZLE

WITH 25MM SWIVEL / MALE BIC OR STORZ COUPLING (OR ALTERNATIVE)

BLACKOUT FD/40 PISTOL GRIP NOZZLE

WITH 40MM SWIVEL / MALE BIC OR STORZ COUPLING (OR ALTERNATIVE)

MATERIALS

| BODY - | BLACK ANODISED ALUMINIUM |
| --- | --- |
| HANDLE / STIRRUP CONTROL - | INJECTION MOULDED BLACK UHMWPE |